MATERIAL SAFETY DATA SHEET

KLEAN & GO

Heavy Duty Stone, Tile and Grout Cleaner

Section I - Product Identification

Product Name: 


Klean & Go, Heavy Duty Stone, Tile & Grout Cleaner

Distributed by:


International Stoneworks, Inc. 2523 Fairway Park, Suite 510, Houston, TX 77092


(713) 956-8291

Section II - Components

If present, IARC, NTP and OSHA carcinogens and chemical subject to the reporting requirements of SARA Title III Section 313 are identified in this Section.

Ingredient

CAS#

% (BY VOL)

OSHA PEL

ACGIH TLV

Sodium Metasilicate
6834-92-0
N/A


N/E


N/E

2-Butoxyethanol
111-76-2
3-6%


25 ppm

N/E

Proprietary Surfactants  N/A

N/A

Non-hazardous proprietary ingredients

Section III - Health Hazard Data

Chronic Effects of Overexposure:





Inhalation:  N/D


Eye Contact:  N/D


Skin Contact:  N/D


Ingestion:  N/D

Acute Effects of Overexposure:



Inhalation:  Excessive exposure may cause respiratory irritation.



Eye Contact:  May cause severe eye irritation


Skin Contact:  Reported to cause a burning and itching at contact site.


Ingestion:  Large amounts may cause gastrointestinal irritation with injury.


Teratogenicity:  Exposure has no known effects on mother.

Section IV - Emergency First Aid Procedures

If Swallowed:
If patient is fully conscious, induce vomiting if large amounts are ingested. Call the poison control center at (800) 764-7661. If symptoms persist, get medical attention.  Do not give anything by mouth to an unconscious person.

Skin Contact:
Flush thoroughly with water for 15 minutes.  If irritation persists, get medical attention.  Launder affected clothing before reuse.

If Inhaled:
Move to fresh air, get medical attention if irritation persists.

Eyes:
 Flush thoroughly with water for 15 minutes, get medical attention.

Section V - Fire and Explosion Data

Flash Point:
No flash point until 100°C

Extinguishing Agents:
   Product is not flammable. Use water, carbon dioxide, dry powder extinguisher, or other media appropriate for materials actually involved in fire.

Autoignition Temp:
None up to 100º C

Klean & Go MSDS

Page 2

Hazardous Decomposition Products:
   Exposure to fire may liberate carbon dioxide, carbon monoxide, water and other unidentified thermal decomposition products from this product or its packaging.

Explosion Data:
Sensitivity to Impact/Static discharge - None

Section VI - Accidental Release Measures

Procedure for Cleanup:
Prevent further spill of material, contain material already spilled.

Section VII - Handling and Storage

Store out of the reach of children and animals.  Keep container closed and store in a cool dry location.  Avoid freezing or extended storage in high temperature. 

Section VIII - Exposure Control and Personal Protection

Ventilation:
Normal room ventilation satisfactory.  Respirator not normally required.

Protective Gloves:
Avoid skin contact.  Wear NBR or rubber gloves for extended or repeated skin contact with concentrate.

Eye Protection:
Avoid eye contact. Wear safety glasses while handling concentrate.

Footwear and Clothing:
Standard work shoes and clothing.

Other:
Do not ingest product.  No other special protective measures required.

Section IX - Physical/Chemical Characteristics

Form/Color:
Liquid/slight yellow


Boiling point:
100°C

Specific Gravity (water = 1):
1.07


Vapor Pressure, mm Hg/20 Deg C:
760 mm/Hg

Evaporation rate (Butyl Acetate = 1):

N/D

pH:
11.5-12.5

Odor:
Mild


Section X - Stability and Reactivity

Stability:
Yes

Incompatibility:  Avoid strong oxidizing materials


Reactivity, and under what conditions:  None known

Section XI - Toxicological Information

Please call the non-emergency number under Section I if information is required.

Section XII - Ecological Information

Please call the non-emergency number under Section I if information is required.

Section XIII - Disposal Considerations

See Section VI for special handling or disposal consideration.

Special disposal considerations:
NA

Klean & Go MSDS

Page 3

Section XIV - Transport Information

For transportation accidents call CHEMTREC® 1-800-424-9300.

Special shipping information:

Non-hazardous.

Section XV - Regulatory Information

N/A

Section XVI - Other Information

Format and Preparation Complies with ANSI Z400.1-1993

Revision Date: 01-01-2001

We believe said statements, technical information and recommendations contained herein are reliable.  Statements contained herein are given without warranty or guarantee of any kind, express or implied, and we assume no responsibility for any loss, damage, or expense, direct or consequential, arising out of their use.

