MASTICE 2000 FOR TRAVERTINE

SAFETY DATA SHEET

COMPILED July 25, 2005
Disclaimer: the Bellinzoni Company provides the information presented below.  International Stoneworks, Inc. declines any responsibility as to the veracity of this information
	1. Product and Company Name

Trade Name:


Mastice 2000 
Trade Code:


009MASTRAS-EN

Product Type and Use:
Polyester mastic for travertine, limestone and other beige natural stones
Seller: Bellinzoni s.r.l.

Via Gallarate 205 - Milano 20151 – Italy / Tel: +390238001051 / Fax: +3901238000481

In the USA: Maverick International Import/Export Group: Tel: 888-307-4628 

	2. Composition/ingredients

 Hazardous components within the meaning of EEC directive 67/548 and corresponding classification:
40% - 50% styrene.

N.67/548/CEE: 601-026-00-0  CAS: 100-42-5   EINECS: 202-851-5

Xn Harmful

R20 Harmful by Inhalation

R36/38 Irritating to eyes and skin.

	3. Potential Hazards

The product is a liquid that can catch fire at temperatures in excess of 21° C if exposed to an ignition source.

The product is harmful following acute exposure to it and poses a serious health threat if inhaled.

If brought into contact with eyes, the product causes irritation that may last over 24 hours, and if brought into contact with the skin it causes significant inflammation with erythema, scabs, and oedema. 

	4. First Aid Measures
Contact with the eyes:

Wash immediately and thoroughly with running water, keeping eyelid raised for at least 10 minutes.  Following this, protect the eyes with sterile gauze or a clean, dry handkerchief.  OBTAIN A MEDICAL EXAMINATION.

Contact with the skin:

Immediately take off all contaminated clothing

Areas of body that have, or are only even suspected of having, come into contact with the product must be rinsed immediately with plenty of running water and possibly with soap.

Swallowing:

Do not under any circumstances induce vomiting.  OBTAIN A MEDICAL EXAMINATION IMMEDIATLEY.   Give liquid paraffin to drink; do not give milk or animal or vegetable fats of any kind.

Inhalation:

Ventilate the premises.  The patient is to be removed immediately from the contaminated premises and made to rest in a well ventilated area. OBTAIN MEDICAL ATTENTION. For emergencies, contact the Poison Control Center at (800) 764-7661.

	Page 2. Mastice 2000 for Travertine Material Safety Data Sheet

5. Fire-Fighting Measures

Recommended extinguishers:

CO2, or Dry chemical fire extinguishers. 

Extinguishers not to be used: 

None in particular.

Risks arising from combustion:

Avoid inhaling the fumes.

Protective Equipment:

Use protection for the respiratory tract.

Cool the containers exposed to the fire and water

	6. Accidental Release Measures

Measures for personal safety:

Use gloves and protective clothing.

Environmental measures:

Limit leakages with earth and sand.

Eliminate all unguarded flames and possible sources of ignition.  Do not smoke.

If the product has escaped into a water course, into the drainage system, or has contaminated the ground or vegetation, notify the competent authorities.

Cleaning Methods:

Rapidly recover the product.  To do so, wear a mask and protective clothing.  If the product is in a liquid form, stop it from entering the drainage system.

Recover the product for re-use if possible, or for elimination.  The product might, where appropriate, be absorbed by inert material. 

After the product has been recovered, rinse the area and materials involved with water.

	7. Handling and Storage

Handling Precautions:

Avoid contact and inhalation of vapors.  See too Paragraph 8 below.

Do not eat or drink while working.  

Do not smoke while working.

Incompatible materials:

None in particular.

Storage Conditions:

Keep away from unguarded flame, sparks, and heat sources.  Avoid direct exposure to sunlight.

Instructions as regards storage premises:

Cool and adequately ventilated. Store out of the reach of children and animals.

	8. Exposure limits and personal protective measures

Precautionary Measures:

Give adequate ventilation to the premises where the product is stored and/or handled.

Respiratory Protection:

Use adequate protective respiratory equipment, e.g. CEN/FFP-2 (S) or CEN/FFP-3(S).

Eye Protection:

Use close-fitting safety goggles and/or visor conforming to BS 2092 GRADE 1.

Page 3. Mastice 2000 for Travertine Material Safety Data Sheet

Protection for Skin:

Use clothing that provides comprehensive protection to the skin, e.g. cotton, rubber, PVC or viton

Exposure Limits (ACGIH):

Styrene

TLV TWA:20 ppm, A4 – 85 mg/m3, A4  TLV STEL: 40 ppm, A4 – 170 mg/m3, A4

	9.Physical and Chemical Properties:

Appearance & Color:        Liquid dense transparent

Odor:                                 Of styrene

Flash Point:                       > 21° C

Relative Density:               1.11

Solubility in Water:           Insoluble

Liquid Solubility:              Insoluble

	10. Stability and Reactivity:

Conditions to avoid:

Stable under normal conditions.

Substance to avoid:

Avoid contact with combustible materials.  The product could catch fire.

Hazardous decomposition products:

None.

	11. Toxicoligical Data

The concentration of each substance should be borne in mind in assessing the toxicological effects deriving from the preparation.

Set out below is the toxicological information relating to the main substances in the preparation.  The product does not contain toxicologically relevant substances.

	12. Ecological Data

Adopt good working practices, so that the product is not released into the environment.

	13. Disposal Measures

Recover if possible.  Send to authorized disposal plants for incineration under controlled conditions.  In so doing, comply with the local and national regulations currently in force.

	14. Transportation Data

Road (ADR):                        3.31 C

Rail (RID):                            3.31 C

Air (ICAO/IATA):               3 UN 3269

Sea (IMO):                           3.3 pag 3372 UN 3269

UN number:                         3269

	15. Regulatory Information

88/379/EEC (Classification and Labeling):

Symbols:  Xn Harmful

R Phrases:  

R10 Flammable

R20 Harmful by inhalation

R36/38 Irritating to eyes and skin.

Page 4. Mastice 2000 for Travertine Material Safety Data Sheet

S Phrases: 

 S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S24/25 Avoid contact with skin and eyes.

Contents: Styrene

Where applicable, refer to the following regulatory provisions: 

Directive 82/501/EEC (‘Activities linked to risks of serious accidents’) and subsequent amendments. 

Where applicable, refer to the following regulatory provisions:

Directive 82/501/EEC (‘Activities linked to risks of serious accidents’) and subsequent amendments.

	16. Other Information

Main bibliographic sources:

ECDIN – Environmental Chemicals Data and Information Network – Joint Research

Centre, Commission of the European Communities.

SAX’s DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS –Eighth Edition – Van Nostrand Reinold.

ACGIH – Threshold Limit Values – 1993/94 Edition.

This information contained herein is based on our state of knowledge at the above-specified date.  It refers solely to the product indicated and constitutes no guarantee of particular quality.  It is the duty of the user to ensure that this information is appropriate and complete with respect to the specific use intended.

This MSDS cancels and replaces any preceding release.


