Material Safety Data Sheet






U.S. Department of Labor
May be used to comply with






Occupational Safety and Health Administration

OSHA’s Hazard Communication Standard





(Non-Mandatory Form)

29 CFR 1910.1200. Standard must be





Form Approved

consulted for specific requirements.






OMB No. 1218-0072

[image: image1.bmp]
IDENTITY (As Used on Label and List)



Note: Blank spaces are not permitted.  If any item is not applicable, or 

FIVEXTRA® PLUS (Marble Polishing Compound)      
no information is available, the space must be marked to indicate that.


Section I

Supplied by






Emergency Telephone Number

INTERNATIONAL STONEWORKS, INC.



(800) 764-7661

Address (Number, Street, City, State, and Zip Code


Telephone Number for Information     Website Address

2523 Fairway Park, Ste 510, Houston, TX 77092


(713) 956 - 8291 


   www.intlstoneworks.com


Date Prepared 
 March 29, 2007


Section II - Hazardous Ingredients/Identity Information

Hazardous Components    


OSHA PEL     ACGIH TLV    Other Limits                       %(optional)

(Specific Chemical Identity: Common Name)


Potassium Tetraoxalate CAS no. 6487-48-5    N/A
N/A
         N/A 

Oxalic Acid

CAS no. 6153-56-6    1 mg/m3
1 mg/m3      2 mg/m3STEL
 

Aluminum Oxide
CAS no. 1344-28-1    15 mg/m3    
10 mg/m3    20 mg/m3STEL

Other inert ingredients 


Aluminum Oxide is on the USEPA SARA list.  No materials are on the CA Prop 65 list.

Section III - Physical/Chemical Characteristics

Boiling Point


N/A



Specific Gravity (H2O = 1)  1 gr/cm3

Vapor Pressure (mm Hg.)

N/A



Melting Point

N/A

Vapor Density (AIR =1)

N/A



Evaporation Rate
N/A
Butyl Acetate = 1)


Solubility in Water
 
PARTIAL


Ph
(when water wet)
approximately 3 - 5

Appearance and Odor

Light beige powder / mild sulfur odor

Section IV - Fire and Explosion Hazard Data

Flash Point (Method Used)

N/A



Flammable Limits
LEL 

UEL  
N/A

Extinguishing Media


N/A

Special Fire Fighting Procedures
Product non-combustible. Use media appropriate for surrounding fire


Unusual Fire and Explosion Hazards
N/A


    OSHA 174, Sept. 1985
FIVEXTRA® PLUS (Marble Polishing Compound)

Section V - Reactivity Data

Stability

Unstable


Conditions to Avoid  

Stable
X





Incompatibility (Materials to Avoid)


                                 Recommend avoiding use with Chlorine based bleaches and disinfectants

Hazardous Decomposition or Byproducts     N/A


Hazardous
May Occur


Conditions to Avoid


Will Not Occur
X

Section VI - Health Hazard Data


Routes of Entry:

Inhalation  


Skin




Ingestion  

Dizziness/Headache

Irritation/Drying


Nausea/Irritation

Health Hazards (Acute and Chronic)




Carcinogenicity


NTP



IARC Monographs


OSHA Regulated

  



NO



NO



NO (nuisance dust only)


Signs and Symptoms of Exposure

Excessive dermal contact may cause skin redness.  Ingestion may cause intestinal pain.  Excessive inhalation may cause difficulty breathing.


Medical Conditions

Generally Aggravated by Exposure

Avoid contact with areas of prior skin burns or lacerations


Emergency and First Aid Procedures:   INHALATION:  Remove victim from area. Provide supplemental breathing air or oxygen. SKIN IRRITATION:  Wash with alkaline soap/ flush with water.  EYE IRRITATION:  Flush with plenty of water for at least 15 minutes/call physician.  INGESTION:  Do not induce vomiting/drink water/call physician.

Section VII - Precautions for Safe Handling and Use

Steps to be Taken in Case Material is Released or Spilled
If dry, pick up with broom and/or shovel.  If wet, pick up with wet/dry vacuum, then flush and clean area with water.  Mop up residual.           


Waste Disposal Method
Do not dispose solids in wastewater drains.  Material may settle out and cause plugging.  Dispose of solids in accordance with local regulations.

Precautions to Be Taken in Handling and Storing  Keep covered and dry until ready for use.  Store away from children & animals.   Keep away from foods and drinks including animal foods.


Other Precautions  This product is to be used exclusively by cleaning and stone restoration professionals.

Please read FIVEXTRA® PLUS information sheet carefully before usage.

Section VIII - Control Measures

Respiratory Protection (Specify Type)
Use NIOSH approved dust protection mask during use.



Ventilation
Local Exhaust
Desirable



Special  NA

Mechanical (General)

NA


Other


Protective Gloves
  Hand protection is recommended



Eye Protection  safety glasses recommended

Other Protective Clothing or Equipment  Water proof apron & slip resistant boots recommended during use.

Work/Hygienic Practices 
Do not eat or drink during application.  Use normal good personal hygiene.

Page 2



U.S. Government Printing Office 1987-181-504/64362

