MATERIAL SAFETY DATA SHEET

SEAL & GO ™ “ENHANCER”

Color Enhancing Impregnator/Sealer

Section I - Product Identification

Product Name: 

   Seal & Go ™ “Enhancer”  Color Enhancing Impregnator/Sealer

Distributed by:


   International Stoneworks Inc. 2523 Fairway Park, Suite 510. Houston, Texas 77092

   (713) 956-8291

Section II - Components

If present, IARC, NTP and OSHA carcinogens and chemical subject to the reporting requirements of SARA title III Section 313 are identified in this Section.

Ingredient

CAS#

% (BY VOL)

OSHA PEL
ACGIH TLV

Synthetic Isoparaffinic

 Hydrocarbon

64742-48-9

75-80%
500 ppm
100 ppm

D-Limonene

5989-27-5

15-20%
N/E

N/E

Fluoropolymer Concentrate 




Proprietary

N/A
Non-hazardous proprietary ingredients.  

Concentrate




N/A

2% - 10%
Non-hazardous proprietary ingredients.

Section III - Health Hazard Data

Chronic Effects of Overexposure:





No known applicable information.


Acute Effects of Overexposure:


Ingestion:
May irritate stomach and intestines. May be aspirated into lungs if swallowed, resulting in pulmonary edema and chemical pneumonitis.

Skin:
Mild irritant.

Inhalation:
May cause headache, dizziness, nausea, unconsciousness.

Eye:
Mild irritant

Section IV - Emergency First Aid Procedures

If Swallowed:
Do not induce vomiting. Rinse mouth with water then drink one glass of water. Seek immediate medical attention.

Skin Contact:
Remove contaminated clothing and launder before reuse. Wash with soap and water.

If Inhaled:
Remove to fresh air. If unconscious, give artificial respiration and seek medical attention immediately.

Eyes:
Immediately flush with large amounts of water for at least 15 minutes and seek medical attention.

Section V - Fire and Explosion Data

Flash Point:
141°F

Method Used:
TCC

Auto Ignition:

N/A
Combustible Liquid Class III-A

Seal & Go ™ “Enhancer”

Page 2

Extinguishing Agents:

Dry Chemical, Carbon Dioxide, Foam or Dry Chemical

Special Fire Fighting Procedures:
Evacuate personnel to safe area. Avoid breathing decomposition products. Shut off source, if possible. Use NIOSH/MSHA approved self-contained breathing apparatus. Do not use water.

Unusual Fire And Explosion Hazards:
A combustible liquid can form combustible mixtures at temperatures at or above flashpoint. Material can accumulate static charges which can cause an incendiary electrical discharge.

Section VI - Accidental Release Measures

Procedure For Cleanup:
Eliminate all ignition sources. Leaks should be stopped. Spills should be contained and cleaned up immediately. Evacuate unnecessary personnel. Large spills should be removed by vacuum truck. Smaller spills may be soaked up with compatible absorbent material, which should be placed in closed containers outdoors. Do not hose  spills down drains, sewers or waterways.

Section VII - Handling and Storage

Keep container closed. Store in a cool, well ventilated place. Do not handle or store near an open flame, heat or other sources of ignition.

Section VIII - Exposure Control and Personal Protection

Respiratory Protection:
Not normally required. If vapor concentration becomes high, use NIOSH approved respirators.

Ventilation:
Local ventilation is adequate.

Protective Gloves:
Solvent resistant gloves should be worn.

Eye Protection:
Chemical safety goggles, plus full face shield when appropriate.

Section IX - Physical/Chemical Characteristics

Form:
Liquid

Appearance:
Slightly Yellow

Specific Gravity:
0.8 @ 25°C

Solubility In Water (% by wt):
< 0.1% @ 68°F

Evaporation (Butyl Acetate = 1):
< 1

Odor:
Citrus

Boiling Point:
340ºF

Water Reactive:
None

Section X - Stability and Reactivity

Stability:
Stable - Polymerization will not occur

Conditions to Avoid:
N/A

Incompatibility (materials to avoid):
Oxygen, strong oxidizing agents

Hazardous Decomposition or Combustion: Carbon oxides formed when burned

Section XI - Toxicological Information

Please call the non-emergency number under Section I if information is required.

Seal & Go ™ “Enhancer”

Page 3

Section XII - Ecological Information

Please call the non-emergency number under Section I if information is required.

Section XIII - Disposal Considerations

See Section VI for special handling or disposal consideration.

Section XIV - Transport Information

For transportation accidents call CHEMTREC® 1-800-424-9300.

Special shipping information:


Petroleum Distillate, N.O.S., Combustible Liquid UN 1268, III

Note: In containers of 119 gallons or less this product is not regulated by DOT.

Section XV - Regulatory Information

N/A

Section XVI - Other Information

Format and Preparation Complies with ANSI Z400.1-1993

Revision Date: 02-26-2001

We believe said statements, technical information and recommendations contained herein are reliable.  Statements contained herein are given without warranty or guarantee of any kind, express or implied, and we assume no responsibility for any loss, damage, or expense, direct or consequential, arising out of their use.

