[image: image1.jpg]INTERNATIONAL STONEWORKS, INC,

(I

(I

2523 FAIRWAY PARK, SUITE 510 * HOUSTON, TEXAS 77092
TEL: (713) 956-8291 * 1-800-775-TILE * FAX: (713) 956-8293
www.intlstoneworks.com * email: sales@intlstoneworks.com





STRIP & GO®                                                          Page 1
MATERIAL SAFETY DATA SHEET
1.  CHEMICAL PRODUCT AND COMPANY IDENTIFICATION
     Product Identification
     ----------------------
     Product Name:  STRIP & GO©
     Company Identification
     ----------------------
     INTERNATIONAL STONEWORKS, INC
     2523 FAIRWAY PARK DRIVE, STE 510
     HOUSTON, TEXAS 77092
     1-713-956-8291 (For product information)
     1-800-764-7661 (For emergencies) 
2.  COMPOSITION/INFORMATION ON INGREDIENTS
     Chemical Name                                Amount              CAS Number
     -------------                                --------            ----------
     2-BUTOXYETHANOL                                < 5%              111-76-2
     (See Section 8 for exposure guidelines)
     (See Section 15 for regulatory information)
     COMPOSITION COMMENT:
     This material or all of its components are listed on the Inventory of 
     Existing Chemical Substances under the Toxic Substance Control Act (TSCA).
     HAZARDS DISCLOSURE
     This product contains hazardous materials as defined by the OSHA Hazard 
     Communication Standard 29 CFR 1910.1200.
STRIP & GO®                                                          Page 2
_______________                                               __________________
     SARA 311 Categories:
          Immediate (Acute) Health Effects....:  Yes
          Delayed (Chronic) Health Effects....:  No
          Fire Hazard.........................:  No
          Sudden Release Of Pressure Hazard...:  No
          Reactivity Hazard...................:  No
3.  HAZARDS IDENTIFICATION
          *****************   EMERGENCY OVERVIEW   *****************
          *                                                        *
          *                        CAUTION                         *
          *                                                        *
          *  Keep away from                                        *
          *
heat, sparks and flames!  May cause skin, eye, or      *
          *  respiratory tract irritation. KEEP AWAY FROM CHILDREN.*                        
          *                                                       ٭
                   **********************************************************
     HMIS Rating -                    Health:  2
                                Flammability:  1
                                  Reactivity:  0
                   Personal Protection Index:  X
     NFPA/HMIS Definitions:  (0-Least, 1-Slight, 2-Moderate, 3-High, 
     4-Extreme).  These values are obtained using the guidelines or published 
     evaluations prepared by the national Fire Protection Association (NFPA) or 
     the National Paint and Coating Association (for HMIS ratings).
     POTENTIAL HEALTH EFFECTS
     EYE:
     Contact may cause eye irritation.
     SKIN:
     May cause skin irritation with discomfort or rash, redness, swelling, and 
     itching.
     INHALATION:
     Inhalation of vapors or mist may cause irritation to respiratory tract.
STRIP & GO®                                                          Page 3
_____________________                                              _____________
     INGESTION:
     Ingestion of larger amounts may cause gastric disturbances.  May cause 
     headaches, dizziness,  nausea, vomiting, diarrhea and 
     general weakness.
     SIGNS AND SYMPTOMS OF EXPOSURE:
     Irritation as noted.
     CARCINOGENICITY INFORMATION:
     N.A.
     MEDICAL CONDITIONS AGRAVATED BY EXPOSURE:
     May aggravate pre-existing eye, skin or respiratory tract conditions upon 
     contact.
4.  FIRST AID MEASURES
     EYE CONTACT FIRST AID:
     Hold eyelids apart and flush eyes with plenty of water for at least 15 
     minutes.  Get immediate medical attention.
     SKIN CONTACT FIRST AID:
     In case of contact, immediately flush skin with plenty of water for at 
     least 15 minutes while removing contaminated clothing and shoes.  Get 
     medical attention if irritation develops or persists.  Wash clothing 
     before reuse.
     INHALATION FIRST AID:
     Remove to fresh air.  If breathing is difficult, give oxygen.  If not 
     breathing, give artificial respiration.  Get immediate medical attention.
     INGESTION FIRST AID:
     If swallowed, immediately give 2 glasses of water.  Get immediate medical 
     attention.  Never give anything by mouth to an unconscious person.
5.  FIRE FIGHTING MEASURES
     FLAMMABLE PROPERTIES
     Flash Point:  < 200 F
     Auto-ignition Temperature:  N/A
STRIP & GO®                                                           Page 4
____________________                                               _____________
     FLAMMABLE LIMITS IN AIR
     N/A
     FLAMMABLE PROPERTIES:
     Use self-contained breathing apparatus and protective equipment.
     EXTINGUISHING MEDIA:
     Water, carbon dioxide, foam or dry powder.
     FIRE & EXPLOSION HAZARDS:
     FIRE FIGHTING INSTRUCTIONS:
     As in any fire, wear self-contained breathing apparatus pressure-demand 
     MSHA/NIOSH (approved or equivalent) and full protective gear.
     COMBUSTION PRODUCTS:
     Burning may produce oxides of carbon.
6.  ACCIDENTAL RELEASE MEASURES
     SAFEGUARDS (PERSONNEL):
     Wear appropriate personal protective equipment.  Eliminate all sources of 
     ignition - heat, sparks, flame, electricity, impact and friction.
     INITIAL CONTAINMENT:
     Treat or dispose of waste material in accordance with all local, 
     state/provincial, and national requirements.
     LARGE SPILLS PROCEDURE:
     Treat or dispose of waste material in accordance with all local, 
     state/provincial, and national requirements.
     SMALL SPILLS PROCEDURE:
     Absorb spills with inert material.  Treat or dispose of waste material in 
     accordance with all local, state/provincial, and national requirements.
     MISCELLANEOUS:
     Flush spill area with water.
STRIP & GO®                                                           Page 5
____________________                                               _____________
7.  HANDLING AND STORAGE
     HANDLING (PERSONNEL):
     Eliminate all sources of ignition - heat, sparks, 
     flame, electricity, impact and friction.  Avoid breathing (dust, vapor, 
     mist, gas).  Do not get in eyes, on skin or clothing.  Wash thoroughly 
     after handling.  Follow all MSDS/label precautions even after container is 
     emptied because they may retain product residues. KEEP AWAY FROM CHILDREN.
     HANDLING (PHYSICAL ASPECTS):
     Eliminate all sources of ignition - heat, sparks, flame, electricity, 
     impact and friction.  Keep container closed to avoid contamination.  Store 
     in a cool dry area. KEEP AWAY FROM CHILDREN
     STORAGE PRECAUTIONS:
     Keep container closed when not in use.  Eliminate all sources of ignition 
· heat, sparks, flame, electricity, impact and friction. KEEP AWAY FROM 

CHILDREN.
8.  EXPOSURE CONTROLS / PERSONAL PROTECTION
     ENGINEERING CONTROLS:
     Use static controls.  Use process enclosures, local exhaust ventilation, 
     or other engineering controls to control airborne levels below recommended 
     exposure limits.
     EYE / FACE PROTECTION REQUIREMENTS:
     Wear safety glasses with side shields (or goggles) and a face shield.
     SKIN PROTECTION REQUIREMENTS:
     Where contact is likely, wear chemical resistant gloves, a chemical suit, 
     rubber boots, and chemical safety goggles plus a face shield.  Nitrile, 
     neoprene or natural rubber gloves.
     RESPIRATORY PROTECTION REQUIREMENTS:
     When there is potential for airborne exposures in excess of applicable 
     limits, wear NIOSH/MSHA approved respiratory protection.
     MISCELLANEOUS:
     Observe the usual precautions for handling any chemical substance.
          EXPOSURE GUIDELINES:
          2-Butoxyethanol
               OSHA TWA:  25 ppm
               ACGIH TWA:  25 ppm
               OSHA STEL:  75 ppm ppm
               Skin Designation:  Yes
STRIP & GO®                                                           Page 6
____________________                                               _____________
9.  PHYSICAL AND CHEMICAL PROPERTIES
     FORM ..................:  Liquid
     COLOR .................:  Colorless/light amber
     ODOR ..................:  Mild
     SOLUBILITY IN WATER ...:  Soluble
     SPECIFIC GRAVITY ......:  1.00 average (Water = 1)
10.  STABILITY AND REACTIVITY
      STABILITY:
      Stable.
      POLYMERIZATION:
      Hazardous polymerization will not occur.
      INCOMPATIBILITY WITH OTHER MATERIALS:
      Avoid contact with strong oxidizing agents.
      DECOMPOSITION:
      Burning may produce oxides of carbon.
11.  TOXICOLOGICAL INFORMATION
12.  ECOLOGICAL INFORMATION
      No information available.
13.  DISPOSAL CONSIDERATIONS
      WASTE DISPOSAL:
      Treat or dispose of waste material in accordance with all local, 
      state/provincial, and national requirements.
14.  TRANSPORTATION INFORMATION
      PRODUCT LABEL..........:  STRIP & GO ®
              ***THIS PRODUCT IS NOT REGULATED BY D.O.T.***
STRIP & GO®                                                          Page 7
____________________                                               _____________
15.  REGULATORY INFORMATION
      REGULATORY DISCLOSURES:
      Chemicals subject to the reporting requirements of SARA Title III Section 
      313 and 40 CFR Part 372:  2-Butoxyethanol (Glycol Ether Category) CAS 
      111-76-2, .
            SARA Title III - Section 313
           2-Butoxyethanol  (111-76-2)

      STATE REGULATIONS:

           2-Butoxyethanol  (111-76-2)

              NEW JERSEY - Workplace Hazard-Environmental Hazard
              PENNSYLVANIA – Workplace Hazard

              MASSACHUSETTS – Hazardous Substance

              NEW YORK – Hazardous Substance

      MISCELLANEOUS INFORMATION:
      This material or all of its components are listed on the Inventory of 
      Existing Chemical Substances under the Toxic Substance Control Act (TSCA).
16.  OTHER INFORMATION
      Prepared By.......:  cjl
      Approved By.......:  
      Title.............:  Environmental Manager
      Approval Date.....:  May, 2009
      Supercedes Date...:  January, 2010
      CAL Number........:  I2857 
      ADDITIONAL INFORMATION:
      The data in this Material Safety Data Sheet relates only to the specific 
      material designated herein.
    **********************************************************************
      This information is furnished without warranty, expressed or implied, 
except that it is accurate to the best Knowledge of INTERNATIONAL STONEWORKS, INC. The data on this sheet are related to the specific material designated herein. INTERNATIONAL STONEWORKS, INC  assumes no legal responsibility for use or reliance upon these data.
    **********************************************************************
                                  END OF MSDS
    ********************************************************************** 
[image: image1.jpg]